Particle-Wise Decomposition of Mixed Wafer Maps by Size-Aware Point-Set Segmentation

[Enter Primary Author's Full Name Here]

[Enter co-author's name/co-author's name here]

[Enter primary author's e-mai/co-author's email here]

[Enter primary author's affiliation name/co-author's affiliation name here]

[Enter primary author's region (Country) here]

Phone: +[Enter Country Code here] -[Enter Phone# here]

1. INTRODUCTION—Particle contamination in semiconductor manufacturing equipment is a major yield concern. Total count is widely monitored, but it cannot distinguish a spatially concentrated event from a random population. Wafer-scale particle distributions can reflect equipment geometry and operating conditions [1,2]. In production, particles from different generation or transport mechanisms may coexist and form overlapping signatures [3]. A wafer-level class, or a multi-label indication of signatures present, does not reveal which measured particles constitute each signature. It therefore cannot directly provide signature-specific count, size, and spatial statistics or link those particle subsets to process, equipment, and maintenance histories. Prior studies address mixed wafer signatures by clustering or rasterized analysis and use full-resolution coordinates for wafer-level recognition [3,4]. We target a complementary output: a spatial-pattern membership label for every original inspection event while retaining continuous coordinates, inspection-reported size, and record identity. The resulting decomposition is an intermediate representation for source investigation, not direct physical root-cause identification.

2. METHOD—A wafer map is represented as an unordered set P = {pᵢ | i = 1, …, N}, with pᵢ = (xᵢ, yᵢ, sᵢ). Here, (xᵢ, yᵢ) is the continuous wafer-plane location and sᵢ is the normalized inspection-reported equivalent size; size is an attribute, not physical height. A PointMLP-based segmentation network [5] predicts one macro spatial-pattern class per event. Because each output remains indexed to its original record, pattern-conditioned counts, size distributions, and spatial distributions are obtained without reconstructing event identity from an image mask. The benchmark contains wafers with one to three coexisting patterns and ground-truth membership for every particle. As an auxiliary experiment, the same particle sets were rasterized for a U-Net with multi-hot pixel targets. Because it predicts pixels and was trained for only 10 epochs, its metrics are not directly comparable with particle-wise results.

3. RESULTS AND DISCUSSION—On the held-out mixed-pattern test set, PointMLP achieved particle-wise accuracy of 0.8441 and mean intersection over union (mIoU) of 0.7151. This demonstrates the feasibility of recovering event-level membership when one to three patterns coexist while preserving each prediction's link to its original record. Engineers can therefore examine the count, size distribution, and location distribution of each predicted population separately. In the auxiliary 10-epoch pixel-wise experiment, U-Net achieved micro/macro IoU of 0.1841/0.2643 and micro/macro F1 of 0.3110/0.3807. These results show non-trivial learning in the rasterized formulation but cannot quantify a PointMLP advantage because prediction units, targets, and training differ. A controlled comparison requires back-mapping image predictions, validation-converged training, and common particle-wise metrics. Independently of scores, the point-set formulation avoids coordinate quantization and preserves event identity by construction.

4. CONCLUSION—We proposed a size-aware point-set framework for decomposing mixed wafer maps into record-linked particle populations. PointMLP achieved particle-wise accuracy of 0.8441 and mIoU of 0.7151 with one to three coexisting patterns. The output supports correlation with equipment states, process conditions, and maintenance histories. Future work will evaluate independently measured mixtures, unknown patterns, tool/process shifts, and a matched particle-wise raster baseline.

REFERENCES—[1] S. H. Park, S. Kim, and J.-G. Baek, Appl. Sci. 8, 224 (2018). [2] C.-M. Ku and S. Cheng, Appl. Sci. 12, 5684 (2022). [3] N. G. Shankar and Z. W. Zhong, Microelectron. J. 36, 718–724 (2005). [4] L. Frittoli et al., Pattern Recognit. 124, 108488 (2022). [5] X. Ma et al., Proc. ICLR (2022).